Job Report

# Front Brake Pad Replacement

Practical Task 5

| NAME | Marcus Reed |
| --- | --- |
| STUDENT ID | 4471029 |
| COURSE | AUT 120 Brakes and Suspension |
| INSTRUCTOR | Ms. Patel |
| SCHOOL | Riverside Community College |
| DATE | October 11, 2026 |

# Vehicle information and customer complaint

| Item | Details |
| --- | --- |
| Year | 2010 |
| Make | Toyota |
| Model | Camry LE, 2.5L four cylinder |
| Mileage | 118,642 miles |

The repair order said the front brakes squealed when stopping, mostly at low speed. The pedal felt normal and the car didn't pull or vibrate. A squeal like that usually means a pad wear indicator is touching the rotor, so I expected low pads before I even pulled a wheel.

# Safety precautions

I wore safety glasses and nitrile gloves.

Fender covers went on first, then a seat cover and floor mat.

I lifted the car at the pinch weld lift points and set the lift locks.

No compressed air on brake dust. I used brake cleaner over a drain pan, since the dust is bad to breathe.

Each caliper hung on a bungee, never by the brake hose.

I kept a rag under the reservoir because brake fluid strips paint.

I pumped the pedal until it was firm before moving the car.

# Tools and equipment

Two-post lift

Breaker bar and ratchet with sockets

Open-end wrench for the slide pins

Click-type torque wrench

Large C-clamp

Outside micrometer (25 to 50 mm) and vernier caliper

Wire brush and brake cleaner

Suction gun and DOT 3 brake fluid

High-temperature brake grease

New ceramic pads with shims and abutment clips

Bungee cord

# Procedure

1.	The fluid was at MAX even with worn pads, so I pulled some out with the suction gun to keep it from overflowing later.

2.	I broke the lug nuts loose on the ground, raised the car and pulled both front wheels.

3.	Holding each slide pin with the wrench, I removed the slide pin bolts and hung the caliper.

4.	I removed the old pads with their shims and clips and measured the pad lining (Figure 1).

INSERT PICTURE HERE Photo of the old pads you removed lying next to the new pads, friction side up, so the difference in thickness is easy to see Click in this box, delete this text, then use Insert > Pictures

Figure 1: Old pads next to the new ones

5.	I measured each rotor at eight spots with the micrometer (Figure 2). Both were over the minimum with no deep grooves, so my instructor OK'd reusing them.

INSERT PICTURE HERE Photo of you measuring the rotor thickness with the micrometer on the area the pads touch, not on the rust lip at the edge Click in this box, delete this text, then use Insert > Pictures

Figure 2: Measuring the left rotor with the micrometer

6.	I took the brackets off, wire brushed them and pulled the slide pins. The right upper pin was dry and sticky, so I cleaned and regreased both pins.

7.	I reinstalled the brackets and torqued the bolts to 79 lb-ft.

8.	New abutment clips went in with a little grease where the pad ears slide.

9.	I pushed the piston back with the C-clamp while watching the reservoir.

10.	I installed the new pads and shims and torqued the slide pin bolts to 25 lb-ft (Figure 3).

INSERT PICTURE HERE Photo of the finished right front brake before the wheel went back on: new pads in, caliper and slide pin bolts installed Click in this box, delete this text, then use Insert > Pictures

Figure 3: Finished right front installation

11.	I put the wheels on, lowered the car and torqued the lug nuts to 76 lb-ft in a star pattern.

12.	I pumped the pedal and topped off the fluid with DOT 3.

13.	On the road test I bedded in the pads with ten moderate stops from 35 mph. No more squeal.

# Measurements

Pad values are lining only. Rotor values are the lowest of eight readings per side.

| Measurement | Left front | Right front | Minimum spec | Result |
| --- | --- | --- | --- | --- |
| Inner pad | 2.5 mm | 1.5 mm | 1.0 mm | Replace |
| Outer pad | 3.5 mm | 3.0 mm | 1.0 mm | Replace |
| Rotor thickness | 26.4 mm | 26.2 mm | 25.0 mm (0.984 in) | Reuse |

On both sides the inner pads wore faster, which is normal since the piston pushes right on them. The right inner pad was a lot thinner though. That matches the sticky slide pin, because a caliper that can't slide leaves the inner pad doing most of the work. Both rotors had at least 1.2 mm left above the minimum cast into the rotor hat.

# Caliper assembly sketch

This Camry has a single piston floating caliper. When the piston pushes the inner pad into the rotor, the caliper body slides inward on its pins and pulls the outer pad against the other side. Figure 4 shows my sketch.

INSERT PICTURE HERE Scan or photo of your hand drawn sketch. Label the caliper body, piston, dust boot, bleeder screw, brake hose, slide pins and boots, slide pin bolts, caliper bracket, inner and outer pads, shims, abutment clips, wear indicator and rotor Click in this box, delete this text, then use Insert > Pictures

Figure 4: Hand drawn sketch of the front caliper assembly

# Reflection

I think the job went well. Everything was torqued to spec and the brakes were quiet and firm on the road test. Pulling the slide pins was worth it, since new pads on a sticky pin would just wear unevenly again.

What I'd do differently is use the micrometer more carefully. My first left rotor reading was 27.6 mm, way too thick for 118,000 miles, because I had it on the rust lip at the edge. Next time I'll measure on the pad contact area from the start and check the micrometer with the 25 mm standard first.